# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 1- INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA1205 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2-INDUSTRY PROBLEM SOLVING TASK |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | MOURISH R | Reg.No.: | 192521447 |
| Department: | B TECH IT | Date: | 18.08.2026 |

### ANNEXURE A

INDUSTRY PROBLEM SOLVING TASK

- Smartphone Performance Optimization

A mobile manufacturer observes lag while switching between apps. Analyze the role of L1, L2, L3 cache and DRAM and propose a hierarchical memory redesign to reduce latency and improve throughput.

Smartphone Performance Optimization

Problem: Lag occurs while switching between apps.

Analysis:

L1 Cache: Very fast; stores frequently used instructions/data.

L2 Cache: Larger than L1 and reduces access to slower memory.

L3 Cache: Shared, larger cache that reduces DRAM accesses.

DRAM: Large capacity but much slower than cache.

Proposed Solution:

Use a larger and efficient L2/L3 cache, keep frequently used app data in cache, and use high-bandwidth, low-latency DRAM for active applications.

Result: Reduced memory latency, faster app switching, and improved throughput.

- Cloud Server Memory Bottleneck

A cloud service provider experiences high latency in database queries. Compare cache hierarchy vs virtual memory paging and recommend improvements using appropriate memory technologies.

Cloud Server Memory Bottleneck

Problem: Database queries have high latency.

Analysis:

Cache hierarchy provides very fast access to frequently used database data.

Virtual memory paging uses disk/SSD when RAM is insufficient, causing much higher latency.

Frequent page faults significantly reduce system throughput.

Proposed Solution:

Increase L3/cache capacity, provide sufficient DRAM, optimize database caching, and reduce dependence on virtual-memory paging.

Result: Faster database queries and higher server throughput.

- Autonomous Vehicle Data Processing

An autonomous vehicle must process sensor data in real time. Evaluate SRAM, DRAM, and MRAM and design a memory hierarchy that maximizes bandwidth and minimizes latency.

Autonomous Vehicle Data Processing

Problem: Sensor data must be processed in real time.

Analysis:

SRAM: Extremely fast and suitable for critical real-time data.

DRAM: Large capacity and high bandwidth for sensor datasets.

MRAM: Fast, non-volatile, and energy-efficient.

Proposed Hierarchy:

CPU → SRAM Cache → MRAM → High-bandwidth DRAM → Storage

Critical sensor/control data is kept in SRAM/MRAM, while large sensor datasets are stored in DRAM.

Result: Low latency, high bandwidth, and reliable real-time processing.

- AI Inference Edge Device

An edge AI device needs fast model loading with low power consumption. Compare NAND Flash, DRAM, and RRAM and recommend an optimized hierarchical memory structure.

AI Inference Edge Device

Problem: Fast model loading with low power consumption is required.

Analysis:

NAND Flash: Non-volatile and high capacity but slower.

DRAM: Fast and suitable for active AI models.

RRAM: Non-volatile, potentially energy-efficient, and suitable for emerging AI memoryarchitectures.

Proposed Hierarchy:

AI Processor → RRAM → DRAM → NAND Flash

Frequently used model parameters can be kept in RRAM/DRAM, while complete models are stored in NAND Flash.

Result: Faster model loading, reduced energy consumption, and improved inference performance.

- High-Performance Gaming Console

A gaming console suffers frame drops during large scene loading. Analyze L3 cache vs DRAM throughput and propose memory hierarchy enhancements.

High-Performance Gaming Console

Problem: Frame drops occur while loading large scenes.

Analysis:

L3 Cache: Very fast but limited in size; useful for frequently reused game data.

DRAM: Much larger and provides high bandwidth for textures, models, and scene data. If large assets cannot fit in cache, the system depends heavily on DRAM.

#### Proposed Solution:

Optimize and increase L3 cache where practical.

Use high-bandwidth DRAM for large game assets.

Use efficient prefetching to move upcoming scene data into cache/DRAM before it is needed.

Result: Faster scene loading, fewer memory stalls, and reduced frame drops.

##### Code of Conduct:

I MOURISH R - 192521447certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: MOURISH R

### MARKS ALLOCATION

|  | Understanding of Industry Problem (20%) | Application of Engineering Concepts (25%) | Solution Design (25%) | Use of Tools (15%) | Analysis (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |

| Q4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Q5 |  |  |  |  |  |

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |