# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 3- CLASS TEST

| Course Code: | CSA1201 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- CLASS TEST |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | MOURISH R | Reg.No.: | 192521447 |
| Department: | B.TECH IT | Date: | 18/08/2026 |

### ANNEXURE A CLASS TEST

- Design a heterogeneous I/O bus architecture comparing USB, PCIe, and Thunderbolt, and construct a scalable communication model for multiple peripherals.

- Construct an interrupt handling framework for a multi-core processor comparing vectored interrupts and shared interrupt lines, and analyze latency impact.

- Design a high-speed NVMe storage subsystem using PCIe lanes, DMA transfers, and memory-mapped registers, and justify performance improvements.

- Construct an I/O scheduling mechanism comparing cycle stealing DMA and burst mode DMA for continuous multimedia streaming.

- Design a multi-level I/O communication architecture combining memory-mapped I/O, interrupt handling, and advanced bus interfaces, and evaluate throughput.

Code of Conduct:

I MOURISH R - 192521447 certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student :  MOURISH   R

### MARKS ALLOCATION

|  | Conceptual Understanding (30%) | Accuracy of Answers (30%) | Problem Solving & Application Skills (20%) | Presentation & Organization (10%) | Completion & Time Management (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Conceptual Understanding | 30% | Demonstrates excellent understanding of all concepts with accurate explanations and applications | Shows good understanding of concepts with minor errors | Shows basic understanding but with some misconceptions | Shows limited understanding with major conceptual errors |
| Accuracy of Answers | 30% | Provides completely correct answers with proper steps, logic, and final solutions | Provides mostly correct answers with few minor mistakes | Provides partially correct answers with missing steps/errors | Provides incorrect or incomplete answers with minimal accuracy |
| Problem Solving & Application Skills | 20% | Applies appropriate methods/techniques effectively and solves complex problems independently | Applies suitable methods with minor guidance/errors | Attempts problem solving but has difficulty applying concepts correctly | Unable to apply concepts or select suitable methods |
| Presentation & Organization | 10% | Answers are well structured, neat, and clearly presented with proper notation/format | Answers are organized with minor presentation issues | Answers lack proper organization and clarity in some areas | Poorly organized answers with unclear presentation |
| Completion & Time Management | 10% | Completes all questions within the allotted time with effective planning | Completes most questions within time | Attempts only part of the test due to time management issues | Very few questions attempted or incomplete submission |

#### ANSWERS

- Heterogeneous I/O Bus Architecture: USB, PCIe and Thunderbolt Architecture

CPU

|

System Bus / Root Complex

|

+	+	+

|	|	|

PCIe	USB	Thunderbolt

|	|	|

GPU / NVMe	Keyboard		External GPU Network Card Mouse	Display

Camera	Storage Comparison

| Feature | USB | PCIe | Thunderbolt |
| --- | --- | --- | --- |
| Main purpose | General peripherals | High-performance internal devices | High-speed external devices |
| Latency | Moderate | Very low | Low |
| Bandwidth | High | Very high | Very high |
| Connection | External | Internal | External |
| Examples | Mouse, keyboard, printer | GPU, NVMe SSD | External GPU, display, SSD |

Scalable Communication Model

A root complex/controller connects different interfaces. Low-speed devices use USB,

high-performance internal devices use PCIe, and high-speed external devices use Thunderbolt. Conclusion: This heterogeneous design provides flexibility and allows peripherals to communicate using the interface most suitable for their bandwidth and latency requirements.

- Interrupt Handling in a Multi-Core Processor Framework

Multiple CPU Cores

+	+	+	+

| Core 0 | Core 1 | Core 2 |

+	+	+	+	+	+	+

|	|	| Interrupt Controller

|

+	+	+

|	|

Vectored Interrupts Shared Interrupt

|	|

Device A/B/C	Multiple Devices Comparison

| Feature | Vectored Interrupt | Shared Interrupt Line |
| --- | --- | --- |
| Interrupt identification | Direct vector | Must identify source |
| Latency | Lower | Higher |

| Feature | Vectored Interrupt | Shared Interrupt Line |
| --- | --- | --- |
| Hardware complexity | Higher | Lower |
| Scalability | Better | Limited |
| CPU overhead | Lower | Higher |

Latency Analysis

With vectored interrupts, the processor directly obtains the address of the appropriate interrupt service routine. Therefore, response time is shorter.

With shared interrupts, several devices use the same interrupt line. The processor must check which device generated the interrupt, increasing latency.

Conclusion: For a multi-core processor, vectored interrupts with an advanced interrupt controller provide better latency and scalability.

- High-Speed NVMe Storage Using PCIe and DMA Architecture

CPU

|

PCIe Root Complex

|

PCIe Lanes

|

NVMe Controller

|

NAND Flash Main Components PCIe lanes:

Provide a high-speed connection between the CPU/system and NVMe controller. Multiple lanes increase available bandwidth.

DMA:

Direct Memory Access allows the NVMe controller to transfer data directly between storage and RAM without requiring the CPU to copy every block.

Memory-Mapped Registers:

The CPU communicates with the NVMe controller through registers mapped into the system's address space.

Data Transfer Application

↓ CPU

↓

NVMe Command

↓

NVMe Controller

↓

DMA Transfer

↓ DRAM

Performance Improvements

- PCIe provides high bandwidth.

- Multiple PCIe lanes increase parallel data transfer.

- DMA reduces CPU involvement.

- Memory-mapped registers provide efficient device control.

- NVMe supports multiple queues and parallel commands.

Conclusion: Combining PCIe + DMA + memory-mapped I/O + NVMe queues significantly improves storage throughput and reduces CPU overhead compared with older storage interfaces.

- I/O Scheduling: Cycle Stealing DMA vs Burst Mode DMA

This is especially important for continuous multimedia streaming, such as video and audio. Comparison

| Feature | Cycle Stealing DMA | Burst Mode DMA |
| --- | --- | --- |
| Transfer method | Transfers one/few memory cycles at a time | Transfers a large block continuously |
| CPU interruption | More frequent | Less frequent |
| CPU availability | Higher | Lower during burst |
| DMA throughput | Moderate | High |
| Multimedia suitability | Good | Excellent for continuous streams |

Cycle Stealing CPU → Memory CPU → Memory DMA → Memory CPU → Memory DMA → Memory

DMA periodically takes control of the memory bus. Burst Mode

CPU → Memory

DMA → Memory → Memory → Memory → Memory CPU → Memory

DMA transfers a large block continuously. Recommendation

For continuous high-bandwidth video/audio streaming, burst mode DMA is generally preferred because it provides high throughput and reduces DMA setup/control overhead.

However, burst size should be controlled so that the CPU and other devices are not blocked for too long.

- Multi-Level I/O Communication Architecture Architecture

CPU

|

+	+	+

|	|

Memory-Mapped I/O  Interrupt

|	|

+	+	+

|

I/O Controller

|

Advanced Bus Interface

| / | ¦ | \ |  |
| --- | --- | --- | --- |
| PCIe |  | USB | Thunderbolt |

|	|	|

NVMe	 Camera	External SSD GPU	Keyboard Display

Three Main Levels

Level 1 – Memory-Mapped I/O

Device registers are assigned memory addresses.

Advantage: CPU can access device registers using normal memory instructions. Level 2 – Interrupt Handling

Devices generate interrupts when they need CPU attention.

Advantage: CPU does not need to continuously poll devices, reducing CPU overhead. Level 3 – Advanced Bus Interfaces

PCIe, USB, and Thunderbolt provide high-speed communication with different types of peripherals.

Advantage: High bandwidth and support for multiple devices. Throughput Evaluation

Throughput can be improved by:

- Using high-bandwidth PCIe for high-speed devices.

- Using DMA for large data transfers.

- Using interrupts instead of continuous polling.

- Using multiple bus lanes where supported.

- Using parallel communication paths.

- Buffering data in main memory. Overall architecture:

CPU → Memory-Mapped I/O → I/O Controller → Interrupt/DMA → PCIe/USB/Thunderbolt → Peripherals