SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA1205 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | MOURISH R | Reg.No.: | 192521447 |
| Department: | B.tech IT | Date: | 25/07/2026 |

ANNEXURE A

- A processor performs arithmetic operations on signed 8-bit integers using 2’s complement representation. Consider two numbers: +45 and −23. Analyze in detail how the system performs both addition and subtraction, including binary conversion, alignment of sign bits, and intermediate steps. Determine whether overflow occurs, and explain how the processor detects overflow and interprets the final result.

- A high-performance computing system replaces a ripple carry adder with a 4-bit carry look-ahead adder (CLA) to reduce computation delay. Given two binary inputs, analyze how generate and propagate signals are formed and how carries are computed in advance. Compare the time delay of CLA with ripple carry addition and justify, with reasoning, why CLA improves speed in modern processors.

- In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two numbers, −6 and +5. Analyze the algorithm step-by-step, including the role of the accumulator, multiplier, and Booth bit, along with arithmetic shifts. Explain how the algorithm efficiently handles signed numbers and reduces the number of addition/subtraction operations compared to traditional multiplication.

- A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both restoring and non-restoring division algorithms. Analyze both methods step-by-step, showing intermediate remainders and quotient formation. Compare the two approaches in terms of number of steps, complexity, and efficiency, and explain why non-restoring division is often preferred in modern systems.

- A scientific application requires precise handling of real numbers using the IEEE 754 standard for floating-point representation. Given a decimal number (e.g., −13.25), analyze how it is represented in both single precision and double precision formats, including sign, exponent, and mantissa fields. Further, explain how floating-point addition is performed between two such numbers, highlighting issues like normalization, rounding, and precision loss.

ANSWER 1

Given: A = +45, B = −23 (8-bit, 2's complement)

Step 1: Binary Conversion

+45 = 00101101

+23 = 00010111

−23 (2's complement of 23):
Invert: 11101000
Add 1: 11101001
So −23 = 11101001

Both numbers are already 8-bit, so no sign alignment/padding is needed.

ADDITION: (+45) + (−23)

00101101 (45)

- 11101001 (-23)

1 00010110

Discard the extra carry-out bit (9th bit) → Result = 00010110 = +22 (decimal)

Check: 45 + (−23) = 22 ✔ Correct

Overflow Check:
Carry into sign bit = 1
Carry out of sign bit = 1
Same value → No Overflow

SUBTRACTION: (+45) − (−23)

Rule: A − B = A + (2's complement of B)
2's complement of (−23) = +23 = 00010111

00101101 (45)

- 00010111 (23)

01000100

Result = 01000100 = +68 (decimal)

Check: 45 − (−23) = 45 + 23 = 68 ✔ Correct

Overflow Check:
Carry into sign bit = 0
Carry out of sign bit = 0
Same value → No Overflow

Overflow Detection Rule (used by processor):

Overflow = (Carry into sign bit) XOR (Carry out of sign bit)

If both carries are equal → No overflow
If both carries differ → Overflow occurred

Alternate Rule:
Overflow occurs only when two numbers of the SAME sign are added and the result has the OPPOSITE sign. Since one operand was positive and one negative in both operations here, overflow is impossible in either case.

Final Result Table:

45 + (−23) = 00010110 = +22, Overflow = No

45 − (−23) = 01000100 = +68, Overflow = No

Both results lie within the valid 8-bit signed range (−128 to +127), so both are correctly represented and the processor's Overflow Flag (V) = 0 for both operations.

2.ANSWER

Given: A 4-bit Carry Look-Ahead Adder (CLA) adds two 4-bit binary numbers A = A₃A₂A₁A₀ and B = B₃B₂B₁B₀, with input carry C₀.

Example Inputs: A = 1011 (11), B = 0110 (6)

So: A₃=1, A₂=0, A₁=1, A₀=1
B₃=0, B₂=1, B₁=1, B₀=0
C₀ (initial carry-in) = 0

Step 1: Generate (G) and Propagate (P) Signals

For each bit position i:

Gᵢ = Aᵢ · Bᵢ (Generate — carry is generated regardless of incoming carry)
Pᵢ = Aᵢ ⊕ Bᵢ (Propagate — carry passes through if there's an incoming carry)

Calculating for each bit:

G₀ = A₀·B₀ = 1·0 = 0
P₀ = A₀⊕B₀ = 1⊕0 = 1

G₁ = A₁·B₁ = 1·1 = 1
P₁ = A₁⊕B₁ = 1⊕1 = 0

G₂ = A₂·B₂ = 0·1 = 0
P₂ = A₂⊕B₂ = 0⊕1 = 1

G₃ = A₃·B₃ = 1·0 = 0
P₃ = A₃⊕B₃ = 1⊕0 = 1

Step 2: Carry Look-Ahead Equations

Instead of waiting for the previous carry (as in ripple carry), CLA computes all carries directly from G, P, and C₀ using these equations:

C₁ = G₀ + (P₀ · C₀)
C₂ = G₁ + (P₁ · G₀) + (P₁ · P₀ · C₀)
C₃ = G₂ + (P₂ · G₁) + (P₂ · P₁ · G₀) + (P₂ · P₁ · P₀ · C₀)
C₄ = G₃ + (P₃ · G₂) + (P₃ · P₂ · G₁) + (P₃ · P₂ · P₁ · G₀) + (P₃ · P₂ · P₁ · P₀ · C₀)

Substituting values (C₀ = 0):

C₁ = G₀ + (P₀·C₀) = 0 + (1·0) = 0

C₂ = G₁ + (P₁·G₀) + (P₁·P₀·C₀)
= 1 + (0·0) + (0·1·0) = 1

C₃ = G₂ + (P₂·G₁) + (P₂·P₁·G₀) + (P₂·P₁·P₀·C₀)
= 0 + (1·1) + (1·0·0) + (1·0·1·0)
= 0 + 1 + 0 + 0 = 1

C₄ = G₃ + (P₃·G₂) + (P₃·P₂·G₁) + (P₃·P₂·P₁·G₀) + (P₃·P₂·P₁·P₀·C₀)
= 0 + (1·0) + (1·1·1) + (1·1·0·0) + (1·1·0·1·0)
= 0 + 0 + 1 + 0 + 0 = 1

So: C₁ = 0, C₂ = 1, C₃ = 1, C₄ = 1 (final carry-out)

Step 3: Sum Calculation

Sᵢ = Pᵢ ⊕ Cᵢ

S₀ = P₀ ⊕ C₀ = 1 ⊕ 0 = 1
S₁ = P₁ ⊕ C₁ = 0 ⊕ 0 = 0
S₂ = P₂ ⊕ C₂ = 1 ⊕ 1 = 0
S₃ = P₃ ⊕ C₃ = 1 ⊕ 1 = 0

Result: S₃S₂S₁S₀ = 0001, with final Carry-out C₄ = 1

So the 5-bit result = 10001 = 17 (decimal)

Check: 11 + 6 = 17 ✔ Correct

Step 4: Delay Comparison — Ripple Carry vs CLA

Ripple Carry Adder (RCA):

- Each full adder must wait for the carry from the previous stage before it can compute its own sum and carry.

- Total delay ≈ n × (delay of one full adder), where n = number of bits.

- For a 4-bit adder: Delay ≈ 4 × 2Δ = 8Δ (approx., where Δ is one gate delay, and each full adder takes ~2 gate delays for carry propagation)

- Delay grows linearly with the number of bits: O(n)

Carry Look-Ahead Adder (CLA):

- All G and P signals are computed simultaneously (in parallel) for every bit — only 1 gate delay.

- All carries (C₁, C₂, C₃, C₄) are computed directly from G, P, C₀ using 2-level AND-OR logic — approximately 2 gate delays, regardless of bit position.

- Final sum bits are computed in 1 more gate delay (XOR).

- Total delay ≈ constant (independent of n, for small/moderate n): O(1) to O(log n) for larger adders (due to fan-in limits requiring hierarchical CLA blocks)

Step 5: Justification — Why CLA is Faster

- Parallel Carry Computation: In RCA, carry Cᵢ₊₁ depends on Cᵢ, which depends on Cᵢ₋₁, and so on — this creates a serial dependency chain. In CLA, every carry is expressed directly in terms of the original inputs (G's, P's, and C₀), removing this chain entirely.

- Reduced Critical Path: The critical path in RCA grows with the number of bits (each stage adds delay). In CLA, the critical path is fixed at about 2–3 gate delays for computing any carry, regardless of position.

- Trade-off: CLA requires more hardware (additional AND/OR gates for the look-ahead logic), so it consumes more chip area and power. But in high-performance processors, speed is critical, and this hardware cost is justified because it allows arithmetic operations to complete in far fewer clock cycles.

- Scalability: For wider adders (16, 32, 64-bit), pure CLA becomes impractical due to fan-in limits, so processors use hierarchical/block CLA (grouping 4-bit CLA blocks together with second-level look-ahead) to retain the speed benefit while keeping gate fan-in manageable.

Conclusion: CLA improves speed because it breaks the sequential carry-dependency of ripple carry addition by pre-computing all carries in parallel from generate/propagate signals, reducing delay from linear (O(n)) to nearly constant, at the cost of additional gate hardware — a trade-off well justified in modern high-speed processors.

3.ANSWER

Given: Multiplicand M = −6, Multiplier Q = +5 (using 5-bit registers)

Step 1: Convert Numbers to 2's Complement (5-bit)

+5 = 00101 → Q = 00101

−6:
6 = 00110
Invert: 11001
Add 1: 11010
So M = 11010 (−6)

−M (needed for subtraction step) = +6 = 00110

Step 2: Initialize Registers

A (Accumulator) = 00000
Q (Multiplier) = 00101
Q₋₁ (Booth bit) = 0
M (Multiplicand) = 11010
Number of iterations = 5 (equal to number of bits)

Step 3: Booth's Algorithm Rule

Examine (Q₀, Q₋₁) at each step:

| Q₀ Q₋₁ | Operation |
| --- | --- |
| 0 0 | No operation, only shift |
| 0 1 | A = A + M, then shift |
| 1 0 | A = A − M, then shift |
| 1 1 | No operation, only shift |

After the operation, perform an arithmetic right shift on (A, Q, Q₋₁) together — sign bit of A is preserved/extended.

Step-by-Step Execution

Iteration 1:
Q₀Q₋₁ = 1,0 → A = A − M = 00000 − 11010 = 00000 + 00110 = 00110
Shift (A,Q,Q₋₁) right (arithmetic):
A = 00011, Q = 00010, Q₋₁ = 1

Iteration 2:
Q₀Q₋₁ = 0,1 → A = A + M = 00011 + 11010 = 11101
Shift right:
A = 11110, Q = 10001, Q₋₁ = 0

Iteration 3:
Q₀Q₋₁ = 1,0 → A = A − M = 11110 − 11010 = 11110 + 00110 = 00100 (carry discarded)
Shift right:
A = 00010, Q = 01000, Q₋₁ = 1

Iteration 4:
Q₀Q₋₁ = 0,1 → A = A + M = 00010 + 11010 = 11100
Shift right:
A = 11110, Q = 00100, Q₋₁ = 0

Iteration 5:
Q₀Q₋₁ = 0,0 → No operation (A unchanged = 11110)
Shift right:
A = 11111, Q = 00010, Q₋₁ = 0

Step 4: Final Result

Combine A and Q (10-bit result):

A Q = 11111 00010

Convert from 2's complement to decimal:
Since MSB = 1 → negative number
Invert: 00000 11101
Add 1: 00000 11110 = 30

So Result = −30

Check: (−6) × (+5) = −30 ✔ Correct

Summary Table of Iterations

| Iteration | Q₀ Q₋₁ | Operation | A (after op) | A,Q,Q₋₁ (after shift) |
| --- | --- | --- | --- | --- |
| 1 | 1,0 | A−M | 00110 | A=00011, Q=00010, Q₋₁=1 |
| 2 | 0,1 | A+M | 11101 | A=11110, Q=10001, Q₋₁=0 |
| 3 | 1,0 | A−M | 00100 | A=00010, Q=01000, Q₋₁=1 |
| 4 | 0,1 | A+M | 11100 | A=11110, Q=00100, Q₋₁=0 |
| 5 | 0,0 | None | 11110 | A=11111, Q=00010, Q₋₁=0 |

Final Product = 1111100010 = −30 (decimal)

Comparison with Traditional (Shift-and-Add) Multiplication

| Aspect | Traditional Multiplication | Booth's Algorithm |
| --- | --- | --- |
| Operation per '1' bit | 1 addition | Addition/Subtraction only at bit transitions |
| Handles negative numbers | Requires separate sign handling | Handles signed numbers natively via 2's complement |
| Number of arithmetic ops | Up to n (for n-bit multiplier) | Often fewer, especially with long runs of 1s/0s |
| Hardware complexity | Simple adder only | Adder + subtractor logic (reused), Booth bit register |

Conclusion: Booth's algorithm improves multiplication efficiency by replacing repeated additions with a smaller number of additions/subtractions triggered only at bit transitions, while natively supporting signed 2's complement arithmetic without extra sign-correction steps — making it well-suited for high-speed processor ALUs.

4.ANSWER

Given: Dividend = 13 (1101), Divisor = 3 (0011), using 4-bit registers (5-bit for Accumulator to track sign)

Expected Result: 13 ÷ 3 = Quotient 4, Remainder 1

METHOD 1: RESTORING DIVISION

Initial Setup:
A (Accumulator) = 00000
Q (Quotient/Dividend) = 1101
M (Divisor) = 00011
n = 4 (number of bits)

Algorithm Rule (each iteration):

- Shift (A, Q) left as a combined register

- A = A − M

- If A is negative (sign bit = 1): set Q₀ = 0, and restore A by A = A + M
If A is non-negative: set Q₀ = 1 (no restore needed)

Iteration 1:
Shift (A,Q) left: A = 00001, Q = 1010
A = A − M = 00001 − 00011 = 11110 (negative)
→ Q₀ = 0, Restore: A = A + M = 11110 + 00011 = 00001
Result: A = 00001, Q = 1010

Iteration 2:
Shift (A,Q) left: A = 00011, Q = 0100
A = A − M = 00011 − 00011 = 00000 (non-negative)
→ Q₀ = 1 (no restore)
Result: A = 00000, Q = 0101

Iteration 3:
Shift (A,Q) left: A = 00000, Q = 1010
A = A − M = 00000 − 00011 = 11101 (negative)
→ Q₀ = 0, Restore: A = A + M = 11101 + 00011 = 00000
Result: A = 00000, Q = 1010

Iteration 4:
Shift (A,Q) left: A = 00001, Q = 0100
A = A − M = 00001 − 00011 = 11110 (negative)
→ Q₀ = 0, Restore: A = A + M = 11110 + 00011 = 00001
Result: A = 00001, Q = 0100

Final Result (Restoring Division):
Quotient (Q) = 0100 = 4
Remainder (A) = 00001 = 1

✔ Check: 13 = (3 × 4) + 1 → Correct

Summary Table:

| Iteration | Shift | A−M Result | Negative? | Action | Q₀ |
| --- | --- | --- | --- | --- | --- |
| 1 | A=00001,Q=1010 | 11110 | Yes | Restore | 0 |
| 2 | A=00011,Q=0100 | 00000 | No | Keep | 1 |
| 3 | A=00000,Q=1010 | 11101 | Yes | Restore | 0 |
| 4 | A=00001,Q=0100 | 11110 | Yes | Restore | 0 |

METHOD 2: NON-RESTORING DIVISION

Initial Setup: Same as above — A = 00000, Q = 1101, M = 00011

Algorithm Rule (each iteration):

- If A is non-negative → shift (A,Q) left, then A = A − M
If A is negative → shift (A,Q) left, then A = A + M

- If resulting A is non-negative → Q₀ = 1; if negative → Q₀ = 0

- No immediate restore step — correction is deferred

- After all iterations, if final A is negative, perform one final correction: A = A + M

Iteration 1:
A is non-negative (0) → Shift (A,Q) left: A = 00001, Q = 1010
A = A − M = 00001 − 00011 = 11110 (negative)
→ Q₀ = 0
Result: A = 11110 (negative), Q = 1010

Iteration 2:
A is negative → Shift (A,Q) left: A = 11101, Q = 0100
A = A + M = 11101 + 00011 = 00000 (non-negative)
→ Q₀ = 1
Result: A = 00000, Q = 0101

Iteration 3:
A is non-negative → Shift (A,Q) left: A = 00000, Q = 1010
A = A − M = 00000 − 00011 = 11101 (negative)
→ Q₀ = 0
Result: A = 11101 (negative), Q = 1010

Iteration 4:
A is negative → Shift (A,Q) left: A = 11011, Q = 0100
A = A + M = 11011 + 00011 = 11110 (negative)
→ Q₀ = 0
Result: A = 11110 (negative), Q = 0100

Final Correction (only because final A is negative):
A = A + M = 11110 + 00011 = 00001

Final Result (Non-Restoring Division):
Quotient (Q) = 0100 = 4
Remainder (A) = 00001 = 1

✔ Check: 13 = (3 × 4) + 1 → Correct (matches restoring method)

Summary Table:

| Iteration | A sign before shift | Shift Result | Operation | New A | Negative? | Q₀ |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Non-neg | A=00001,Q=1010 | A−M | 11110 | Yes | 0 |
| 2 | Negative | A=11101,Q=0100 | A+M | 00000 | No | 1 |
| 3 | Non-neg | A=00000,Q=1010 | A−M | 11101 | Yes | 0 |
| 4 | Negative | A=11011,Q=0100 | A+M | 11110 | Yes | 0 |
| Final correction | — | — | A+M | 00001 | — | — |

COMPARISON: Restoring vs Non-Restoring Division

| Criterion | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Operations per iteration | Subtract, then conditionally add back (up to 2 operations/step) | Exactly 1 operation per step (either add OR subtract, never both) |
| Total additions/subtractions (4-bit example) | 4 subtractions + 3 restores = 7 operations | 4 operations + 1 final correction = 5 operations |
| Decision complexity | Must check sign AND perform restore immediately if negative | Only checks sign to decide next operation (add/subtract) — no immediate restore |
| Hardware control logic | Simpler conceptually, but requires extra add-back cycle, increasing latency | Slightly more complex control (must "remember" sign for next step) but avoids redundant operations |
| Speed | Slower — restore step adds extra clock cycles whenever remainder is negative | Faster — no wasted restore cycles; every step performs useful work |
| Final correction needed | No (restoring happens every step) | Yes, but only once at the very end (if final A is negative) |

Conclusion: Both algorithms produce identical, correct results (Quotient = 4, Remainder = 1 for 13÷3), but non-restoring division achieves this with fewer total arithmetic operations by deferring the correction step to the end rather than restoring after every negative result — making it faster and more hardware-efficient, which is why it is widely adopted in modern processor ALU designs.

5.ANSWER

IEEE 754 Representation of −13.25

Step 1: Convert 13.25 to Binary

Integer part (13): 13 = 1101 Fractional part (0.25): 0.25 = 0.01 (since 0.25 = 1/4 = 2⁻²)

So: 13.25 = 1101.01 in binary

Step 2: Normalize (Scientific Binary Notation)

1101.01 = 1.10101 × 2³

- Mantissa (fraction) = 10101 (bits after the binary point)

- Exponent = 3

- Sign = negative (since the number is −13.25) → Sign bit = 1

SINGLE PRECISION (32-bit: 1 sign + 8 exponent + 23 mantissa)

Sign bit: 1 (negative)

Exponent (biased, bias = 127): Biased exponent = 3 + 127 = 130 130 in binary (8 bits) = 10000010

Mantissa (23 bits, fractional part only, leading 1 implied/hidden): 10101 followed by zeros to fill 23 bits: 10101000000000000000000

Final 32-bit representation:

| Sign | Exponent (8 bits) | Mantissa (23 bits) |
| --- | --- | --- |
| 1 | 10000010 | 10101000000000000000000 |

Full bit string: 1 10000010 10101000000000000000000

DOUBLE PRECISION (64-bit: 1 sign + 11 exponent + 52 mantissa)

Sign bit: 1 (negative)

Exponent (biased, bias = 1023): Biased exponent = 3 + 1023 = 1026 1026 in binary (11 bits) = 10000000010

Mantissa (52 bits): 10101 followed by zeros to fill 52 bits: 10101000000000000000000000000000000000000000000000 (padded to 52 bits)

Final 64-bit representation:

| Sign | Exponent (11 bits) | Mantissa (52 bits) |
| --- | --- | --- |
| 1 | 10000000010 | 1010100000000000000000000000000000000000000000000000 |

Key Point: Double precision uses more exponent bits (wider range) and more mantissa bits (higher precision), allowing it to represent a much larger range of values with greater accuracy than single precision — critical for scientific computing where rounding errors accumulate over many operations.

Floating-Point Addition: Step-by-Step Analysis

Example: Add (−13.25) + (6.5)

Step 1: Represent Both Numbers in Normalized Binary Scientific Form

−13.25 = −1.10101 × 2³ (from above)

6.5 = 110.1 (binary) = 1.101 × 2²

Step 2: Align Exponents (Equalize Exponents Before Adding Mantissas)

Since floating-point addition requires equal exponents, the number with the smaller exponent must be adjusted to match the larger exponent.

- Larger exponent = 3 (from −13.25)

- Smaller exponent = 2 (from 6.5) → needs to increase by 1

To increase the exponent of 6.5 by 1, shift its mantissa right by 1 bit (this is the alignment/denormalization step):

1.101 × 2² → 0.1101 × 2³

(Note: shifting right means losing the "normalized" form of 1.xxx temporarily — this is expected during alignment)

Step 3: Perform Mantissa Addition (with Aligned Exponents)

Now both numbers have exponent = 3:

−1.10101 × 2³ +0.11010 × 2³ (6.5 aligned, one extra bit shown for precision during calculation — this extra bit is called a guard bit)

Since signs differ (one negative, one positive), this is effectively subtraction of magnitudes:

1.10101 − 0.11010

0.11011

Result = 0.11011 × 2³, and since the larger-magnitude operand (13.25) was negative, the final result is negative:

Result = −0.11011 × 2³

Step 4: Normalize the Result

0.11011 × 2³ is not normalized (leading bit before the binary point must be 1, i.e., form must be 1.xxxxx).

Shift left by 1 bit and decrease exponent by 1:

0.11011 × 2³ = 1.1011 × 2²

So: Result = −1.1011 × 2²

Step 5: Convert Back to Decimal (Verification)

−1.1011 × 2² = −110.11 (binary)

= −(4+2+0+.5+.25) = −6.75

Check: (−13.25) + 6.5 = −6.75 ✔ Correct

Key Issues in Floating-Point Addition

1. Alignment (Exponent Equalization)

Before mantissas can be added, exponents must match. This requires shifting the mantissa of the number with the smaller exponent to the right, which reduces its effective precision since bits shifted out (or shifted beyond the register width) may be lost.

2. Normalization After Addition

After the raw addition/subtraction, the result may not be in the standard 1.xxxxx form:

- If the result has leading zeros (like our example, 0.11011), it must be shifted left, decreasing the exponent — this is called normalization.

- If the result overflows the mantissa field (e.g., produces a carry into a new digit like 10.xxx), it must be shifted right, increasing the exponent.

3. Rounding

Since the mantissa field has a fixed number of bits (23 for single, 52 for double), any bits generated beyond this limit during alignment or normalization must be rounded. IEEE 754 defines several rounding modes:

- Round to nearest, ties to even (default and most common)

- Round toward zero

- Round toward +∞ or −∞

Rounding introduces small errors, especially when adding numbers of very different magnitudes.

4. Precision Loss (Catastrophic Cancellation & Guard Bits)

- When two numbers of similar magnitude but opposite sign are added (as in our example, effectively 13.25 − 6.5), significant bits can cancel out, leaving fewer meaningful digits in the result — this is called catastrophic cancellation.

- To minimize this, hardware typically keeps a few extra bits (guard bits, round bit, and sticky bit — together called GRS bits) during intermediate calculation steps before final rounding, improving accuracy.

- When numbers have very different exponents (e.g., adding 1.0 × 2¹⁰⁰ and 1.0 × 2⁰), the smaller number's mantissa may be shifted so far right that it becomes effectively zero after alignment — this is a real precision limitation in floating-point systems, sometimes called "absorption."

Summary Table

| Aspect | Single Precision | Double Precision |
| --- | --- | --- |
| Total bits | 32 | 64 |
| Sign bits | 1 | 1 |
| Exponent bits | 8 (bias 127) | 11 (bias 1023) |
| Mantissa bits | 23 | 52 |
| −13.25 representation | 1 10000010 10101000000000000000000 | 1 10000000010 1010100000...0 (52 bits) |
| Precision (~decimal digits) | ~7 digits | ~15-16 digits |

Conclusion: IEEE 754 provides a standardized way to represent real numbers with a trade-off between range (exponent) and precision (mantissa). Floating-point addition requires careful exponent alignment, mantissa addition/subtraction, normalization, and rounding — each step introducing potential precision loss, which is why numerical algorithms in scientific computing must be designed with awareness of these floating-point limitations (e.g., avoiding subtraction of nearly equal large numbers, using double precision when higher accuracy is required).

Code of Conduct:

I MOURISH R-192521447 Nocertify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: MOURISH R

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |