SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA1205 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL 2- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | MOURISH R | Reg.No.: | 192521447 |
| Department: | B.Tech IT | Date: | 25/07/2026 |

ANNEXURE A

1. A Booth's multiplication program produces an incorrect product for the inputs -13 × 9. Debug the algorithm by identifying the incorrect shift or addition/subtraction step and optimize the implementation to reduce execution cycles.

2. A processor implementing the restoring division algorithm generates an incorrect quotient for 29 ÷ 5. Debug the iteration sequence, identify the faulty restore operation, and propose an optimized correction.

3. During the execution of the non-restoring division algorithm, the quotient bits are incorrect after the fourth iteration. Identify the logic error, correct the algorithm, and verify the final quotient and remainder.

4. A Booth multiplier performs arithmetic right shifts as logical shifts, resulting in an incorrect signed multiplication result. Debug the issue and explain how arithmetic shifting improves correctness.

5. A floating-point addition program based on the IEEE 754 single-precision standard gives an incorrect result because the exponents are not aligned properly. Identify the bug and optimize the alignment process

ANSWER 1

Debugging Booth's Multiplication Algorithm for -13 × 9

Booth's multiplication algorithm should produce the correct result for -13 × 9, which is -117. If the program generates an incorrect product, the error is most likely caused by an incorrect addition/subtraction operation or an incorrect arithmetic right shift during one of the iterations.

In Booth's algorithm, the operation performed depends on the values of Q₀ and Q₋₁:

- Q₀Q₋₁ = 00 → No operation (only arithmetic right shift)

- Q₀Q₋₁ = 01 → A = A + M

- Q₀Q₋₁ = 10 → A = A − M

- Q₀Q₋₁ = 11 → No operation (only arithmetic right shift)

A common mistake is performing A = A + M instead of A = A − M when Q₀Q₋₁ = 10. Since the multiplicand is a negative number (-13), this incorrect operation changes the accumulator value and produces an incorrect final product.

Another common error is using a logical right shift instead of an arithmetic right shift. In Booth's algorithm, an arithmetic right shift must preserve the sign bit (most significant bit). If a logical right shift is used, the sign bit becomes 0, changing the negative value into a positive value and resulting in an incorrect multiplication result. Therefore, the arithmetic right shift must always copy the sign bit to maintain the correct signed value.

After correcting the addition/subtraction operation and performing the arithmetic right shift correctly in every iteration, Booth's algorithm produces the correct result:

-13 × 9 = -117

Optimization to Reduce Execution Cycles

The implementation can be optimized by reducing unnecessary operations. When Q₀Q₋₁ is 00 or 11, no addition or subtraction is required, so only the arithmetic right shift is performed. This reduces the number of arithmetic operations. The addition/subtraction and shift operations can also be combined efficiently within each iteration to reduce instruction count. Using registers instead of repeated memory accesses further improves execution speed. These optimizations decrease the total execution cycles while maintaining the correctness of the multiplication.

Final Answer

The incorrect product is caused by either performing the wrong arithmetic operation (A + M instead of A − M when Q₀Q₋₁ = 10) or by using a logical right shift instead of an arithmetic right shift. Correcting these errors ensures that Booth's algorithm produces the correct product -117. The implementation can be optimized by skipping unnecessary addition/subtraction operations for 00 and 11, combining arithmetic and shift operations efficiently, and using register-based execution to reduce the overall execution cycles.

2.ANSWER

Debugging the Restoring Division Algorithm for 29 ÷ 5

The restoring division algorithm should produce the correct result for 29 ÷ 5, where the quotient is 5 and the remainder is 4. If the processor generates an incorrect quotient, the error is most likely caused by a faulty restore operation after subtraction.

In the restoring division algorithm, the divisor is subtracted from the accumulator after every left shift. If the subtraction result is positive or zero, the quotient bit is set to 1. If the subtraction result is negative, the previous value of the accumulator must be restored by adding the divisor back, and the quotient bit is set to 0.

A common error occurs when the processor fails to restore the accumulator after a negative subtraction result. Instead of adding the divisor back, the negative value is retained, causing incorrect values to be used in the following iterations. As a result, the quotient bits are generated incorrectly, producing an incorrect final quotient.

For the division 29 ÷ 5, the correct final result is:

- Dividend = 29

- Divisor = 5

- Quotient = 5

- Remainder = 4

Optimization to Reduce Execution Cycles

The restoring division algorithm can be optimized by eliminating unnecessary restore operations. Instead of restoring the accumulator after every negative subtraction, the processor can use the non-restoring division approach, where the next iteration performs an addition or subtraction based on the previous result. This reduces the number of arithmetic operations and improves execution speed. Additionally, using efficient register operations and minimizing memory access further reduces execution cycles.

Final Answer

The incorrect quotient is caused by a faulty restore operation, where the accumulator is not restored after a negative subtraction. The correct implementation must add the divisor back to the accumulator whenever the subtraction result is negative and set the quotient bit to 0. After correcting the restore operation, the restoring division algorithm correctly produces the result:

29 ÷ 5 = Quotient 5, Remainder 4.

The implementation can be optimized by using the non-restoring division algorithm, reducing unnecessary restore operations, and using efficient register-based execution to decrease the overall execution cycles.

3.ANSWER

Debugging the Non-Restoring Division Algorithm

In the non-restoring division algorithm, the quotient bits are determined based on the sign of the accumulator (A) after each arithmetic operation. If the quotient bits become incorrect after the fourth iteration, the most likely cause is an error in the decision logic used to update the accumulator or set the quotient bit.

The correct logic of the non-restoring division algorithm is as follows:

- If the accumulator (A) is positive or zero, subtract the divisor (A = A − M) and set the quotient bit to 1.

- If the accumulator (A) is negative, add the divisor (A = A + M) and set the quotient bit to 0.

- After every operation, perform a left shift of the accumulator and quotient registers before proceeding to the next iteration.

- At the end of all iterations, if the accumulator is negative, restore it by adding the divisor once to obtain the correct remainder.

The logic error after the fourth iteration occurs when the algorithm subtracts the divisor even though the accumulator is negative, or sets the quotient bit to 1 instead of 0. This causes all subsequent quotient bits to be incorrect because the accumulator carries an invalid value into the remaining iterations.

To correct the algorithm, the processor must check the sign of the accumulator after each iteration. If the accumulator is negative, it must perform addition in the next iteration and set the quotient bit to 0. If the accumulator is positive or zero, it must perform subtraction and set the quotient bit to 1. After completing all iterations, if the accumulator remains negative, the divisor should be added once to restore the correct remainder.

Verification of Final Result

After correcting the logic error, the non-restoring division algorithm produces the correct result:

- Quotient = 5

- Remainder = 4

Final Answer

The incorrect quotient bits after the fourth iteration are caused by an error in the decision logic, where the processor performs the wrong arithmetic operation or sets the wrong quotient bit based on the sign of the accumulator. The algorithm is corrected by performing addition when the accumulator is negative, subtraction when it is positive or zero, and restoring the accumulator at the end if it remains negative. After applying these corrections, the final output is:

Quotient = 5
Remainder = 4

4.ANSWER

Debugging Booth's Multiplication Algorithm

Booth's multiplication algorithm is designed to perform signed binary multiplication. If the Booth multiplier performs logical right shifts instead of arithmetic right shifts, the signed multiplication result becomes incorrect because the sign bit is not preserved during shifting.

In an arithmetic right shift, the most significant bit (MSB), which represents the sign of the number, is copied into the new leftmost position. This preserves the sign of both positive and negative numbers throughout the multiplication process. In contrast, a logical right shift always inserts 0 into the leftmost bit, changing a negative number into a positive value and producing an incorrect product.

The logic error occurs when the processor performs a logical right shift after each addition or subtraction step. As a result, the accumulator loses its sign information, and the subsequent arithmetic operations are performed on incorrect values. This causes the final multiplication result to be incorrect, especially when one or both operands are negative.

To correct the algorithm, the logical right shift must be replaced with an arithmetic right shift. During each shift operation, the sign bit (MSB) should be copied into the newly created leftmost position while shifting the accumulator, multiplier, and the extra bit (Q-1) together. This ensures that the signed value is maintained correctly throughout all iterations.

How Arithmetic Right Shift Improves Correctness

- Preserves the sign bit (MSB) of signed binary numbers.

- Maintains the correct two's complement representation of negative values.

- Prevents negative numbers from becoming positive during shifting.

- Ensures correct addition and subtraction in every iteration.

- Produces the correct signed multiplication result for both positive and negative operands.

Final Answer

The error occurs because the Booth multiplier performs a logical right shift instead of an arithmetic right shift. A logical shift inserts 0 into the MSB, causing the sign of negative numbers to change and resulting in an incorrect product. The algorithm is corrected by using an arithmetic right shift, which copies the sign bit into the MSB during every shift. This preserves the signed two's complement representation, ensures correct intermediate calculations, and produces the correct signed multiplication result.

5.ANSWER

Debugging IEEE 754 Single-Precision Floating-Point Addition

In the IEEE 754 single-precision floating-point addition algorithm, both operands must have the same exponent before their mantissas (significands) can be added. If the program produces an incorrect result, the most likely bug is that the exponents are not aligned correctly before performing the addition.

The correct procedure is to compare the exponents of the two floating-point numbers. The number with the smaller exponent must have its mantissa shifted to the right until both exponents become equal. If this alignment step is skipped or performed incorrectly, the mantissas are added with different exponent values, producing an incorrect floating-point result.

The logic error occurs when the program directly adds the mantissas without first aligning the exponents, or when it shifts the mantissa of the larger exponent instead of the smaller one. This changes the numerical value of the operands and results in an incorrect sum.

To correct the algorithm, the processor must first compare the exponents, identify the operand with the smaller exponent, and repeatedly right-shift its mantissa while increasing its exponent until both exponents are equal. After alignment, the mantissas are added or subtracted depending on their signs. Finally, the result is normalized and rounded according to the IEEE 754 standard.

Optimization of the Alignment Process

The alignment process can be optimized by:

- Comparing the exponents only once at the beginning of the operation.

- Shifting only the mantissa of the operand with the smaller exponent.

- Using a barrel shifter to perform multiple-bit shifts in a single clock cycle instead of shifting one bit at a time.

- Skipping the alignment step when both exponents are already equal.

- Performing normalization and rounding only after the addition is completed.

Final Answer

The bug occurs because the floating-point addition program does not align the exponents correctly before adding the mantissas. The mantissa of the operand with the smaller exponent must be right-shifted until both exponents are equal. After proper alignment, the mantissas are added, the result is normalized, and rounding is applied according to the IEEE 754 standard. The alignment process can be optimized by using a barrel shifter, shifting only the smaller mantissa, avoiding unnecessary shifts when exponents are already equal, and performing normalization only once after the addition.

Code of Conduct:

I MOURISH R - 192521447 certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: MOURISH R

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |