SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: | MOURISH R | Reg.No.: | 192521447 |
| Department: | B.Tech IT | Date: | 30/07/2026 |

ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

- Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

1.ANSWER

Aim

To design a processor organization using a 5-stage pipelined datapath, identify the three types of pipeline hazards (data, control, and structural), and compare superscalar techniques such as Out-of-Order Execution, Speculative Execution, and Simultaneous Multithreading (SMT).

1. Processor Organization

A processor organization consists of a Control Unit, Register File, ALU, Memory Unit, and Pipeline Registers. The processor executes multiple instructions simultaneously by dividing instruction execution into different pipeline stages.

Five Pipeline Stages

- IF (Instruction Fetch) – Fetches the instruction from instruction memory.

- ID (Instruction Decode) – Decodes the instruction and reads operands from the register file.

- EX (Execute) – Performs arithmetic and logical operations using the ALU.

- MEM (Memory Access) – Reads from or writes to data memory.

- WB (Write Back) – Writes the result back to the register file.

2. Designed Processor Organization

+----------------------+

|  Instruction Memory  |

+----------+-----------+

|

v

+-------------+

| IF (Fetch)  |

+-------------+

|

v

+-------------+

| ID (Decode) |

+-------------+

|

+-----------------+-----------------+

|                                   |

v                                   v

+-------------------+             +----------------------+

|   Register File   |             | Hazard Detection Unit|

+-------------------+             +----------------------+

|                                   |

+-----------------+-----------------+

|

v

+-------------+

| EX (ALU)    |

+-------------+

|

v

+-------------+

| MEM Stage   |

+-------------+

|

v

+-------------+

| WB Stage    |

+-------------+

|

v

Updated Register File

3. Pipelined Datapath

Instruction

Memory

│

▼

+-------+

|  IF   |

+-------+

│

▼

+-------+

|  ID   |

+-------+

│

▼

+-------+

|  EX   |

+-------+

│

▼

+-------+

| MEM   |

+-------+

│

▼

+-------+

|  WB   |

+-------+

Each stage executes a different instruction simultaneously, thereby improving instruction throughput.

4. Pipeline Hazards

A. Data Hazard

A data hazard occurs when an instruction depends on the result of a previous instruction that has not yet completed.

Example

I1 : ADD R1, R2, R3

I2 : SUB R4, R1, R5

The second instruction requires the updated value of R1, which is still being computed.

Solution

- Forwarding (Bypassing)

- Pipeline Stall (Bubble)

B. Control Hazard

A control hazard occurs due to branch or jump instructions, making it uncertain which instruction should be fetched next.

Example

BEQ R1, R2, LABEL

The processor does not know the next instruction until the branch condition is evaluated.

Solution

- Branch Prediction

- Pipeline Flush

- Delayed Branching

C. Structural Hazard

A structural hazard occurs when two instructions require the same hardware resource simultaneously.

Example

Instruction Fetch and Data Memory Access use the same memory at the same time.

Solution

- Separate Instruction and Data Memory

- Duplicate Hardware Resources

- Pipeline Stall

- Hazard Summary

| Hazard Type | Cause | Example | Solution |
| --- | --- | --- | --- |
| Data Hazard | Dependency between instructions | ADD followed by SUB | Forwarding, Stall |
| Control Hazard | Branch/Jump instruction | BEQ, JMP | Branch Prediction, Flush |
| Structural Hazard | Hardware resource conflict | Shared Memory | Separate Memory, Stall |

6. Superscalar Techniques

A. Out-of-Order Execution

The processor executes independent instructions before earlier stalled instructions to improve CPU utilization.

Advantages

- Reduces pipeline stalls.

- Increases instruction-level parallelism.

- Improves processor performance.

Disadvantages

- Complex hardware.

- Higher power consumption.

B. Speculative Execution

The processor predicts the outcome of branch instructions and executes instructions before the actual branch result is known.

Advantages

- Reduces branch delay.

- Improves execution speed.

- Better pipeline utilization.

Disadvantages

- Wrong predictions waste processing time.

- Additional hardware complexity.

C. Simultaneous Multithreading (SMT)

The processor executes instructions from multiple threads simultaneously using the same CPU core.

Advantages

- Better utilization of execution units.

- Higher throughput.

- Improved multitasking performance.

Disadvantages

- Shared resources may create contention.

- Increased processor complexity.

7. Comparison of Superscalar Techniques

| Feature | Out-of-Order Execution | Speculative Execution | SMT |
| --- | --- | --- | --- |
| Purpose | Executes independent instructions early | Executes predicted instructions | Executes multiple threads simultaneously |
| Branch Prediction | No | Yes | No |
| Multiple Threads | No | No | Yes |
| Performance | High | High | Very High |
| Hardware Complexity | High | Medium | High |
| Main Benefit | Reduces instruction stalls | Reduces branch penalty | Improves CPU utilization |

8. Advantages of the Designed Processor

- Increases instruction throughput.

- Improves processor performance.

- Minimizes execution delays.

- Efficient utilization of CPU resources.

- Supports parallel execution of instructions.

- Reduces pipeline stalls using hazard handling techniques.

9. Applications

- Modern Intel and AMD Processors

- ARM-based Mobile Processors

- High-Performance Computing (HPC)

- Artificial Intelligence Systems

- Cloud Computing Servers

- Embedded Systems

10. Conclusion

The designed processor organization uses a 5-stage pipelined datapath (IF, ID, EX, MEM, WB) to execute multiple instructions simultaneously and improve processor performance. Pipeline hazards such as Data, Control, and Structural Hazards are minimized using forwarding, branch prediction, and separate hardware resources. Superscalar techniques including Out-of-Order Execution, Speculative Execution, and Simultaneous Multithreading (SMT) further enhance performance by increasing instruction-level and thread-level parallelism. This processor organization provides high throughput, efficient resource utilization, and improved overall execution speed, making it suitable for modern computing systems.

2.ANSWER

Aim

To create a model of a computer system using Single-Bus and Multi-Bus organization, perform data transfer operations between registers, memory, and I/O, compare their execution time and efficiency, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

1. Introduction

A bus organization is a communication pathway that transfers data between the processor, memory, and input/output (I/O) devices. Depending on the number of buses used, a computer system can be classified as:

- Single-Bus Organization

- Multi-Bus Organization

2. Single-Bus Organization

In a single-bus organization, one common bus is shared by all processor registers, memory, and I/O devices. Only one data transfer can occur at a time.

Block Diagram

+----------------+

|      CPU       |

| +------------+ |

| | Registers  | |

| |    ALU     | |

| +------------+ |

+-------+--------+

|

========= BUS =========

/            |           \

/             |            \

+---------------+ +-----------+ +-----------+

|    Memory     | | Input Unit| |Output Unit|

+---------------+ +-----------+ +-----------+

3. Multi-Bus Organization

In a multi-bus organization, the processor uses multiple buses to transfer data simultaneously. Different buses connect registers, memory, and I/O independently, reducing waiting time.

Block Diagram

+----------------------+

|        CPU           |

| +------------------+ |

| | Register File    | |

| | ALU              | |

| +------------------+ |

+----+---------+------+

|         |

Bus A|         |Bus B

|         |

+-------+         +-------+

|                         |

+---------------+         +--------------+

|    Memory     |         |  I/O Devices |

+---------------+         +--------------+

|

Bus C

|

Secondary Storage

4. Data Transfer Operations

A. Register to Register Transfer

Example:

R1 ← R2

Data stored in Register R2 is transferred to Register R1.

B. Memory to Register Transfer

Example:

R1 ← M[200]

The data stored at memory address 200 is transferred to Register R1.

C. Register to Memory Transfer

Example:

M[300] ← R2

The data stored in Register R2 is written into memory location 300.

D. I/O to Register Transfer

Example:

R3 ← Input Device

The processor reads data from an input device and stores it in Register R3.

E. Register to Output Device

Example:

Output Device ← R4

The contents of Register R4 are transferred to the output device.

5. Working of Single-Bus Organization

Example Operations

- Memory → Register

- Register → ALU

- ALU → Register

- Register → Output

Since only one bus is available, each transfer occurs one after another.

Estimated Time

| Operation | Clock Cycle |
| --- | --- |
| Memory → Register | 1 |
| Register → ALU | 1 |
| ALU → Register | 1 |
| Register → Output | 1 |
| Total | 4 Cycles |

6. Working of Multi-Bus Organization

Multiple buses allow several transfers to occur simultaneously.

Example:

- Bus A transfers Memory → Register.

- Bus B transfers Register → ALU.

- Bus C transfers ALU → Memory.

These operations occur in parallel.

Estimated Time

| Operation | Clock Cycle |
| --- | --- |
| Parallel Transfers | 1 |
| Remaining Transfer | 1 |
| Total | 2 Cycles |

7. Comparison of Single-Bus and Multi-Bus Organization

| Feature | Single-Bus Organization | Multi-Bus Organization |
| --- | --- | --- |
| Number of Buses | One | Two or More |
| Data Transfer | One transfer at a time | Multiple simultaneous transfers |
| Parallel Processing | No | Yes |
| Execution Time | Higher | Lower |
| Performance | Moderate | High |
| Hardware Cost | Low | High |
| Bus Contention | High | Very Low |
| Efficiency | Lower | Higher |

8. Time and Efficiency Analysis

Assume four data transfers are required.

Single-Bus

- Total Clock Cycles = 4

- Execution Time = Higher

- Throughput = Lower

Multi-Bus

- Total Clock Cycles = 2

- Execution Time = Lower

- Throughput = Higher

Therefore,

Efficiency Improvement

- Approximately 50% reduction in execution time.

- Better utilization of processor resources.

- Increased overall system performance.

9. Advantages of Multi-Bus Organization

- Supports parallel data transfer.

- Reduces processor waiting time.

- Eliminates bus bottlenecks.

- Improves CPU throughput.

- Faster communication between CPU, Memory, and I/O.

- Suitable for high-performance processors.

10. Applications

- Modern Intel and AMD Processors

- ARM-based Mobile Processors

- High-Performance Computing (HPC)

- Graphics Processing Units (GPU)

- Embedded Systems

- Cloud Computing Servers

11. Conclusion

The designed computer system demonstrates both Single-Bus and Multi-Bus organizations. In the Single-Bus Organization, only one transfer occurs at a time, resulting in higher execution time and lower efficiency due to bus contention. In the Multi-Bus Organization, multiple buses enable simultaneous data transfers between registers, memory, and I/O devices, significantly reducing bottlenecks and improving overall processor performance. Therefore, Multi-Bus Organization is more efficient and is widely used in modern computer architectures for high-speed data communication.

3.ANSWER

Aim

To simulate a 5-stage instruction pipeline consisting of Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), and Write Back (WB), execute a set of instructions, measure speedup and throughput, and compare the performance with a non-pipelined processor.

1. Introduction

Instruction pipelining is a technique used to improve processor performance by dividing instruction execution into multiple stages. Multiple instructions are processed simultaneously, with each stage handling a different instruction in the same clock cycle. This increases throughput and reduces the average execution time per instruction.

2. Five Pipeline Stages

| Stage | Description |
| --- | --- |
| IF (Instruction Fetch) | Fetches the instruction from memory. |
| ID (Instruction Decode) | Decodes the instruction and reads operands from registers. |
| EX (Execute) | Performs arithmetic or logical operations using the ALU. |
| MEM (Memory Access) | Reads from or writes to data memory if required. |
| WB (Write Back) | Writes the result back to the register file. |

3. 5-Stage Pipeline Architecture

+----------------------+

| Instruction Memory   |

+----------+-----------+

|

v

+--------------+

| IF (Fetch)   |

+--------------+

|

v

+--------------+

| ID (Decode)  |

+--------------+

|

v

+--------------+

| EX (Execute) |

+--------------+

|

v

+--------------+

| MEM (Memory) |

+--------------+

|

v

+--------------+

| WB (WriteBack)|

+--------------+

|

v

Register File

4. Instruction Set

Assume the following instructions:

I1 : LOAD R1, M[100]

I2 : ADD R2, R1, R3

I3 : SUB R4, R2, R5

I4 : AND R6, R4, R7

I5 : OR  R8, R6, R9

5. Pipeline Execution Table

| Clock Cycle | IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- | --- |
| 1 | I1 | - | - | - | - |
| 2 | I2 | I1 | - | - | - |
| 3 | I3 | I2 | I1 | - | - |
| 4 | I4 | I3 | I2 | I1 | - |
| 5 | I5 | I4 | I3 | I2 | I1 |
| 6 | - | I5 | I4 | I3 | I2 |
| 7 | - | - | I5 | I4 | I3 |
| 8 | - | - | - | I5 | I4 |
| 9 | - | - | - | - | I5 |

6. Non-Pipelined Execution

In a non-pipelined processor, each instruction completes all five stages before the next instruction begins.

| Instruction | Clock Cycles Required |
| --- | --- |
| I1 | 5 |
| I2 | 5 |
| I3 | 5 |
| I4 | 5 |
| I5 | 5 |
| Total | 25 Cycles |

7. Pipelined Execution

For a 5-stage pipeline:

Total Clock Cycles = Number of Instructions + Pipeline Stages − 1

= 5 + 5 − 1 = 9 Clock Cycles

| Processor | Clock Cycles |
| --- | --- |
| Non-Pipelined | 25 |
| Pipelined | 9 |

8. Performance Measurement

A. Speedup

Formula

Speedup = Time taken by Non-Pipelined Processor

------------------------------------

Time taken by Pipelined Processor

Calculation

Speedup = 25 / 9

= 2.78

Speedup = 2.78 times

B. Throughput

Formula

Throughput = Number of Instructions

----------------------

Total Clock Cycles

Non-Pipelined

Throughput = 5 / 25

= 0.20 Instructions/Cycle

Pipelined

Throughput = 5 / 9

= 0.56 Instructions/Cycle

9. Performance Comparison

| Parameter | Non-Pipelined | Pipelined |
| --- | --- | --- |
| Number of Instructions | 5 | 5 |
| Clock Cycles | 25 | 9 |
| Speedup | 1× | 2.78× |
| Throughput | 0.20 Instructions/Cycle | 0.56 Instructions/Cycle |
| CPU Utilization | Low | High |
| Parallel Execution | No | Yes |
| Performance | Lower | Higher |

10. Advantages of Instruction Pipelining

- Executes multiple instructions simultaneously.

- Increases instruction throughput.

- Reduces total execution time.

- Improves CPU utilization.

- Enhances overall processor performance.

- Supports efficient parallel processing.

11. Applications

- Intel and AMD Processors

- ARM-based Mobile Processors

- Embedded Systems

- High-Performance Computing (HPC)

- Graphics Processing Units (GPU)

- Cloud Computing Servers

12. Conclusion

A 5-stage instruction pipeline improves processor performance by overlapping the execution of multiple instructions across the Fetch, Decode, Execute, Memory, and Write Back stages. In this simulation, the pipelined processor completed 5 instructions in 9 clock cycles, while the non-pipelined processor required 25 clock cycles. The pipelined system achieved a speedup of approximately 2.78× and increased the throughput from 0.20 to 0.56 instructions per cycle, demonstrating that pipelining significantly improves execution efficiency and processor performance.

4.ANSWER

Aim

To develop a 5-stage pipeline execution scenario where Data Hazards, Control Hazards, and Structural Hazards occur, observe their impact on instruction execution, and implement Stalling, Data Forwarding, and Branch Prediction techniques to resolve these hazards and improve processor performance.

1. Introduction

In a pipelined processor, multiple instructions execute simultaneously in different pipeline stages. However, conflicts called pipeline hazards may occur, causing delays and reducing processor performance. These hazards are classified into:

- Data Hazard

- Control Hazard

- Structural Hazard

To improve performance, hazard resolution techniques such as Stalling, Data Forwarding, and Branch Prediction are used.

2. Five Pipeline Stages

| Stage | Description |
| --- | --- |
| IF (Instruction Fetch) | Fetch instruction from memory |
| ID (Instruction Decode) | Decode instruction and read registers |
| EX (Execute) | Perform ALU operation |
| MEM (Memory Access) | Read/write memory |
| WB (Write Back) | Store result into register |

3. Pipeline Architecture

+----------------------+

| Instruction Memory   |

+----------+-----------+

|

v

+-------------+

| IF (Fetch)  |

+-------------+

|

v

+-------------+

| ID (Decode) |

+-------------+

|

v

+-------------+

| EX (ALU)    |

+-------------+

|

v

+-------------+

| MEM Stage   |

+-------------+

|

v

+-------------+

| WB Stage    |

+-------------+

|

v

Register File

4. Pipeline Execution Scenario

Assume the following instruction sequence:

I1 : LOAD R1, M[100]

I2 : ADD  R2, R1, R3

I3 : SUB  R4, R2, R5

I4 : BEQ  R4, R0, LABEL

I5 : OR   R6, R7, R8

5. Data Hazard

A Data Hazard occurs when an instruction depends on the result of a previous instruction that has not yet completed.

Example

I1 : LOAD R1, M[100]

I2 : ADD  R2, R1, R3

Here, I2 requires the value of R1, which is still being loaded by I1.

Impact

- Pipeline stall

- Increased execution time

- Reduced throughput

Solution – Data Forwarding

The result from the EX/MEM stage is forwarded directly to the next instruction without waiting for WB.

6. Control Hazard

A Control Hazard occurs because of branch instructions.

Example

I4 : BEQ R4, R0, LABEL

The processor cannot determine the next instruction until the branch condition is evaluated.

Impact

- Incorrect instruction fetch

- Pipeline flush

- Wasted clock cycles

Solution – Branch Prediction

The processor predicts whether the branch is taken or not taken.

If the prediction is correct:

- No pipeline stall

- Faster execution

If incorrect:

- Flush incorrect instructions

- Fetch the correct instruction

7. Structural Hazard

A Structural Hazard occurs when multiple instructions require the same hardware resource simultaneously.

Example

- Instruction Fetch and Data Memory Access attempt to use the same memory in the same clock cycle.

Impact

- Resource conflict

- Pipeline stall

- Reduced processor utilization

Solution

- Separate Instruction Memory and Data Memory

- Duplicate hardware resources

- Stall only when necessary

8. Pipeline Execution Without Hazard Resolution

| Cycle | IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- | --- |
| 1 | I1 | - | - | - | - |
| 2 | I2 | I1 | - | - | - |
| 3 | I3 | I2 | I1 | - | - |
| 4 | I4 | I3 | I2 | I1 | - |
| 5 | I5 | I4 | I3 | I2 | I1 |
| 6 | Stall | I5 | I4 | I3 | I2 |
| 7 | Stall | Stall | I5 | I4 | I3 |
| 8 | - | Stall | Stall | I5 | I4 |
| 9 | - | - | Stall | Stall | I5 |
| 10 | - | - | - | Stall | - |

9. Pipeline Execution After Hazard Resolution

| Cycle | IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- | --- |
| 1 | I1 | - | - | - | - |
| 2 | I2 | I1 | - | - | - |
| 3 | I3 | I2 | I1 | - | - |
| 4 | I4 | I3 | I2 | I1 | - |
| 5 | I5 | I4 | I3 | I2 | I1 |
| 6 | - | I5 | I4 | I3 | I2 |
| 7 | - | - | I5 | I4 | I3 |
| 8 | - | - | - | I5 | I4 |
| 9 | - | - | - | - | I5 |

10. Hazard Resolution Techniques

| Hazard Type | Cause | Impact | Resolution Technique |
| --- | --- | --- | --- |
| Data Hazard | Dependency between instructions | Pipeline stall | Data Forwarding, Stall |
| Control Hazard | Branch instruction | Incorrect instruction fetch | Branch Prediction |
| Structural Hazard | Shared hardware resource | Resource conflict | Separate Memory, Duplicate Resources |

11. Performance Comparison

| Parameter | Without Hazard Resolution | With Hazard Resolution |
| --- | --- | --- |
| Clock Cycles | 10 | 9 |
| Pipeline Stalls | More | Fewer |
| Throughput | Lower | Higher |
| CPU Utilization | Low | High |
| Execution Time | Higher | Lower |
| Overall Performance | Poor | Improved |

12. Advantages of Hazard Resolution

- Reduces pipeline stalls.

- Improves instruction throughput.

- Increases processor efficiency.

- Minimizes branch penalties.

- Enhances CPU resource utilization.

- Improves overall execution speed.

13. Applications

- Intel and AMD Processors

- ARM-based Mobile Processors

- Embedded Systems

- High-Performance Computing (HPC)

- Cloud Computing Servers

- Artificial Intelligence Systems

14. Conclusion

A pipeline execution scenario was developed to demonstrate Data Hazards, Control Hazards, and Structural Hazards in a 5-stage instruction pipeline. These hazards negatively affect processor performance by introducing stalls and execution delays. By implementing Data Forwarding for data hazards, Branch Prediction for control hazards, and Separate Hardware Resources for structural hazards, the processor significantly reduces bottlenecks, improves throughput, minimizes execution time, and achieves better overall performance. This demonstrates the effectiveness of hazard resolution techniques in modern pipelined processors.

5.ANSWER

Aim

To analyze a processor supporting Superscalar Execution, including Out-of-Order Execution and Speculative Execution, extend the study to Hyper-Threading and Simultaneous Multithreading (SMT), and evaluate how multiple instructions are executed in parallel to improve instruction throughput and resource utilization.

1. Introduction

Modern processors improve performance by executing multiple instructions simultaneously. Instead of executing one instruction at a time, advanced processors use Superscalar Architecture, Out-of-Order Execution, Speculative Execution, Hyper-Threading, and Simultaneous Multithreading (SMT) to maximize CPU performance and resource utilization.

2. Processor Organization

The processor consists of multiple execution units that allow several instructions to execute in parallel.

Processor Architecture

+----------------------+

| Instruction Fetch    |

+----------+-----------+

|

v

+----------------------+

| Instruction Decode   |

+----------+-----------+

|

v

+--------------------------+

| Instruction Scheduler    |

+-----------+--------------+

|

+-------------------+-------------------+

|                   |                   |

v                   v                   v

+----------+       +----------+       +----------+

| Integer  |       | Floating |       | Load/    |

|   ALU    |       | Point    |       | Store    |

+----------+       +----------+       +----------+

\                  |                  /

\                 |                 /

+----------------+----------------+

|

v

Write Back Stage

3. Superscalar Execution

A Superscalar Processor issues and executes multiple instructions in the same clock cycle using multiple execution units.

Example

Clock Cycle 1 : ADD   SUB

Clock Cycle 2 : MUL   LOAD

Clock Cycle 3 : STORE AND

Advantages

- Executes multiple instructions simultaneously.

- Improves processor throughput.

- Better utilization of CPU resources.

4. Out-of-Order Execution

Out-of-Order Execution allows the processor to execute independent instructions before earlier stalled instructions.

Example

I1 : LOAD R1, M[100]

I2 : ADD  R2, R1, R3

I3 : MUL  R6, R7, R8

- I2 depends on I1 and must wait.

- I3 is independent and executes before I2.

Advantages

- Reduces pipeline stalls.

- Increases instruction-level parallelism.

- Improves processor performance.

5. Speculative Execution

Speculative Execution predicts the outcome of branch instructions and executes instructions before the branch result is confirmed.

Example

BEQ R1, R2, LABEL

If the branch prediction is correct:

- Instructions continue without interruption.

If incorrect:

- Incorrect instructions are discarded.

- Correct instructions are fetched.

Advantages

- Reduces branch delay.

- Improves pipeline efficiency.

- Increases execution speed.

6. Hyper-Threading

Hyper-Threading enables a single physical CPU core to execute two logical threads simultaneously.

Example

Core 1

├── Thread 1

└── Thread 2

Advantages

- Better CPU utilization.

- Faster multitasking.

- Improved overall performance.

7. Simultaneous Multithreading (SMT)

SMT allows multiple threads to share the processor's execution units during the same clock cycle.

Example

Execution Unit 1 ← Thread A

Execution Unit 2 ← Thread B

Execution Unit 3 ← Thread C

Advantages

- Executes multiple threads in parallel.

- Reduces idle execution units.

- Improves throughput.

8. Parallel Instruction Execution

Example instruction sequence:

I1 : ADD  R1, R2, R3

I2 : MUL  R4, R5, R6

I3 : LOAD R7, M[100]

I4 : SUB  R8, R9, R10

Normal Processor

| Clock Cycle | Executed Instruction |
| --- | --- |
| 1 | I1 |
| 2 | I2 |
| 3 | I3 |
| 4 | I4 |

Total = 4 Clock Cycles

Superscalar Processor

| Clock Cycle | Executed Instructions |
| --- | --- |
| 1 | I1, I2 |
| 2 | I3, I4 |

Total = 2 Clock Cycles

9. Performance Evaluation

Normal Processor

- Instructions = 4

- Clock Cycles = 4

- Throughput = 4 / 4 = 1 Instruction/Cycle

Superscalar Processor

- Instructions = 4

- Clock Cycles = 2

- Throughput = 4 / 2 = 2 Instructions/Cycle

10. Performance Comparison

| Feature | Normal Processor | Superscalar Processor |
| --- | --- | --- |
| Instructions per Cycle | 1 | 2 or More |
| Out-of-Order Execution | No | Yes |
| Speculative Execution | No | Yes |
| Hyper-Threading | No | Yes |
| SMT Support | No | Yes |
| Throughput | Low | High |
| Resource Utilization | Moderate | Excellent |
| Overall Performance | Lower | Higher |

11. Advantages of Superscalar Techniques

- Executes multiple instructions simultaneously.

- Reduces execution time.

- Improves instruction throughput.

- Minimizes pipeline stalls.

- Maximizes CPU resource utilization.

- Enhances multitasking performance.

- Supports high-performance computing applications.

12. Applications

- Intel Core Processors

- AMD Ryzen Processors

- ARM Cortex Processors

- High-Performance Computing (HPC)

- Artificial Intelligence Systems

- Cloud Computing Servers

- Graphics Processing Units (GPU)

13. Conclusion

A processor supporting Superscalar Execution significantly improves performance by executing multiple instructions in parallel during the same clock cycle. Out-of-Order Execution minimizes stalls by executing independent instructions first, while Speculative Execution reduces branch delays through branch prediction. Hyper-Threading enables one physical core to run multiple logical threads, and Simultaneous Multithreading (SMT) allows several threads to share execution resources efficiently. Compared with a conventional processor, the superscalar processor achieves higher instruction throughput, better resource utilization, and reduced execution time, making it the preferred architecture for modern high-performance computing systems.

Code of Conduct:

I MOURISH R - 192521447 certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: MOURISH R

MARKS ALLOCATION

|  | Understanding of Concepts (20%) | Experimental Design (20%) | Use of Tools (25%) | Analysis of Results (20%) | Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |